Robert Dinh

NFC Inventory Management System

Status

/1 Hardware present?

/1 Title Page

/1 Declaration of Joint Authorship

/1 Proposal (500 words)

/1 Executive Summary

# Declaration of Joint Authorship

We, Jonathan Luong, Robert Dinh, and Collin LeDonne, confirm that this work submitted is the joint work of our group and is expressed our own words.  Any uses made within it of the works of any other author, in any form (ideas, equations, figures, texts, tables, programs), are properly acknowledged at the point of use. A list of the references used is included. The work breakdown is as follows: Each of us provided functioning, documented hardware for a sensor or effector. Student A provided the Adafruit HD44780 LCD. Student B provided the Adafruit PN532 NFC/RFID Controller. Student C provided the Adafruit VCNL 4010 Proximity Sensor. In the integration effort Collin LeDonne is the lead for further development of our mobile application, Robert Dinh is the lead for the Hardware, and Jonathan Luong is the lead for connecting the two via the Database.

# Proposal

We have created a mobile application, worked with databases, completed a software engineering course, and prototyped a small embedded system with a custom PCB as well as an enclosure (3D printed/laser cut). Our Internet of Things (IoT) capstone project uses a distributed computing model of a smart phone application, a database accessible via the internet, an enterprise wireless (capable of storing certificates) connected embedded system prototype with a custom PCB as well as an enclosure (3D printed/laser cut), and are documented via this technical report targeting OACETT certification guidelines.

Intended project key component descriptions and part numbers:
Development platform: Raspberry Pi 3 B+
Sensor/Effector 1: Adafruit HD44780 LCD
Sensor/Effector 2: Adafruit PN532 NFC/RFID Controller
Sensor/Effector 3: Adafruit VCNL 4010 Proximity Sensor

We will continue to develop skills to configure operating systems, networks, and embedded systems using these key components to create a functional device that will trigger a proximity sensor to enable a RFID controller in read mode and turn on an LCD display. This device will be used to read an NFC NDEF message from a phone application to identify the student and their requested parts. The Android application that we will develop will allow students to select the parts they need and the application will also generate the NDEF message to be scanned over NFC using dedicated protocols. Also, a web browser application will also be developed mainly to monitor requests and inventory. It will also act as an alternative method for requesting parts. These three components will create an inventory management system that will benefit both the students and the staff. It will seamlessly allow students to request items and pick their parts up without the hassle of purchasing bags and RFID tags for individual students.

Our project description/specifications will be reviewed by, Professor Kristian Medri and Vlad Porcila (Both within the Faculty of Applied Sciences and Technology at Humber College), ideally an employer in a position to potentially hire once we graduate. They will also ideally attend the ICT Capstone Expo to see the outcome and be eligible to apply for NSERC funded extension projects. This typically means that they are from a Canadian company that has been revenue generating for a minimum of two years and have a minimum of two full time employees.

The small physical prototypes that we build are to be small and safe enough to be brought to class every week as well as be worked on at home. In alignment with the space below the tray in the Humber North Campus Electronics Parts kit the overall project maximum dimensions are 12 13/16" x 6" x 2 7/8" = 32.5cm x 15.25cm x 7.25cm.

Keeping safety and Z462 in mind, the highest AC voltage that will be used is 16Vrms from a wall adapter from which +/- 15V or as high as 45 VDC can be obtained. Maximum power consumption will not exceed 20 Watts. We are working with prototypes and that prototypes are not to be left powered unattended despite the connectivity that we develop.

# Executive Summary

The purpose of this document is to lay out the requirements and specifications for the hardware and the client-side application for the NFC Inventory Management System. This system will work in conjunction with an online management interface to maintain inventory in an efficient and organized manner. This product will mainly be developed as a Parts Crib solution at Humber College North Campus but it can also be utilized by other companies for their own purposes.  This product will allow the client’s customers(students) to request items through a mobile or online interface and sign the items out with their phones as a form of identification through NFC. This removes the hassle of providing a card as a form of ID and eliminates the use of RFID tags that have to be individually programmed for each person.  All the data that needs to be organized and managed to make the system work flawlessly, will be handled by the database with an online interface, which allow clients to audit their inventory and manage their customer’s information.

Contents

Declaration of Joint Authorship	3

Proposal	5

Executive Summary	7

List of Figures	13

1.0	Introduction	15

1.1 Product Scope	17

1.2 Requirements and Specifications	18

2.0	Background	19

3.0 Methodology	21

3.1 Required Resources	21

3.1.1 Parts, Components, Materials	21

3.1.2 PCB and Enclosure Design	23

3.1.3 Tools and Facilities	24

3.1.4 Shipping, duty, taxes	26

3.1.5 Time expenditure	27

3.2 Development Platform	28

3.2.1 Mobile Application	28

3.2.2 Image/firmware	38

3.2.3 Connectivity	39

3.2.4 Breadboard/Independent PCBs	41

3.2.4 Printed Circuit Board	48

3.2.5 Enclosure	48

3.3 Integration	48

3.3.1 Enterprise Wireless Connectivity	49

3.3.2 Database Configuration	49

3.3.3 Security	49

3.3.4 Testing	49

4.0 Results and Discussions	51

5.0 Conclusions	53

6.0 References	55

7.0 Appendix	57

7.1 Firmware code	57

7.1.1 main.c File Reference	57

Functions	57

Detailed Description	58

Function Documentation	58

main.c	58

7.1.2 NDEFData.c File Reference	60

Functions	60

Variables	60

Detailed Description	60

Function Documentation	61

NDEFData.c	61

NDEFData.h File Reference	65

Detailed Description	65

NDEFData.h	65

7.1.3 Presence.c File Reference	66

Functions	66

Detailed Description	66

Function Documentation	66

Presence.c	67

Presence.h File Reference	69

Data Structures	69

Typedefs	69

Detailed Description	69

Presence.h	69

7.1.4 Lcd.py File Reference	70

7.2 Application code	71

# List of Figures

Figure 1. Main Activity	29

Figure 2. Create Account Screen	29

Figure 3. Login Screen	30

Figure 4. Home Screen	31

Figure 5. Navigation Drawer	31

Figure 6. Items Category Screen	32

Figure 7. Items Name Screen	33

Figure 8. Item Description Screen	33

Figure 9. Cart Screen	34

Figure 10. Orders Screen	35

Figure 11. Data Visualization	36

Figure 12. LCD Schematic	42

Figure 13. Proximity/Luminosity Sensor	43

Figure 14. NFC Controller Schematic	44

Figure 15. Schematic for all devices wired together	45

Figure 16. Breadboard Layout. VCNL4010(Top), LCD(Bottom), PN532(Right)	46

Figure 17. PCB Design	49

Figure 18 Top of PCB	49

Figure 19  Bottom of PCB	50

Figure 20 Top of PCB after soldering	50

Figure 21. Bottom of PCB after soldering	51

Figure 22. Completed PCB with connected components	52

# Introduction

Report

/1 Hardware present?

/1 Introduction (500 words)

/1 Scope and Requirements

/1 Background (500 words)

/1 References

The idea for this project is to create a solution to the current sign in and out feature at the parts crib at Humber College. The current system has the user download buggy software that is not very intuitive and rather difficult to use without instruction. One must download the software from a dropbox and then the software may need to be loaded several times to work properly.

Our goal is to create a solution to this problem by creating a smart phone application that uses HTTP protocol to send and receive data from a server along with hardware to make the process of borrowing parts easier. The server currently being used is the Humber Apollo Server (apollo.humber.ca) under a student’s profile for testing, but can be moved elsewhere for deployment elsewhere. The mySQL database on the server has 3 dedicated tables for accounts, inventory, and transactions. Each table has one field or more that will be able to relate to another table.

The Android application is designed to interact with the database. It has a graphical user interface to allow students to create a user, sign in, request items and to view previous requests. The process has the user queue the parts they require from the mobile application, which operates like an e-commerce store, which will then notify the employee at the parts crib via web application. When the user queues up parts, this will create a unique NFC code for the user to scan at the parts crib. The employee will then prepare the order for the user to pick-up. When the user arrives at the parts crib, they will scan their phone on the NFC reader which will then update the status of the unique transaction on the database that the employee can see. This will then update the order status on the database until the user scans their phone again on the NFC reader when items have been returned.

The web application will give the employee at the parts crib the ability to release orders, see the contents of the orders, and any relevant information requiring the order. The hardware includes a VCNL 4010 proximity sensor, Adafruit PN532 NFC Sensor, and a 16x2 HD44780 LCD Display. The NFC reader is responsible for receiving the information from the users phone, the LCD display is responsible for displaying the information from the NFC reader to the user, and the proximity sensor will turn up the brightness of the LCD screen when the user puts their phone near the NFC reader. We will be designing a PCB with the intention that the sensors will be placed next to each other to reduce the size of the case. All the components will fit into a case that will be laser cut and the layout of the components will be important so it will be easier to use and understand.

## 1.1 Product Scope

The purpose of our project is to create an inventory management system that permits equipment provisioning for the Parts Crib that provides a friendly user experience for students and employees. The process will interact with a database through requests and finalize transactions through an interaction between a user’s mobile device and our NFC reader located at the crib. The system will implement functionality from the current system as well as provide features for both parties to stay informed regarding items pertaining to them to ensure users are updated accordingly per use.

The database will contain user account information and inventory that range from various tools and equipment. The goal of our project is to create an easy experience for the users of our product and intuitive enough for new users so it is simple and convenient for students to check out equipment.

Our IoT capstone project will implement a distributed computing model comprised of a smart phone application, and a database accessible via PHP scripts using a web browser or our NFC reader embedded system prototype with a custom PCB along with enclosure. The process will be documented via an OACETT certification acceptable technical report.

The project will be limited to the database we have created and will only be tested through simulation of the process. Requiring the Parts Crib for our test is not necessary if we are able to simulate high volume periods. Also, because the application will have personal information of the user there are security clearances that we do not have but we will be implementing secure practices in the handling of our account database involving unique IDs and hashed passwords (Hasing Security, 2019).

## 1.2 Requirements and Specifications

Application

- developed for android mobile devices starting at API 21

- requires internet connection over WiFi

- mobile device must be NFC-equipped

Hardware

- Requires wall outlet for power

- Wi-Fi to communicate with database

- Raspberry Pi 3 B+ with WiFi and I2C enabled

Database

- MySQL Database to read/write/store data

- Requires use of PHP scripts and/or web browser to communicate to the database stored on student account in Humber College servers

# Background

Radio frequency identification, also known as RFID, is the utilization of tags with integrated chips and coiled wire that can be activated using radio signals to retrieve their unique identification number or other information. Tags that are often passive, do not require a power source as its main source is from a reader that sends out an electromagnetic signal (Butterfield & Szymanski, 2018). These RFID tags are commonly found in everyday activities such as tapping a membership card to get into a car wash, to make a payment for food or to get a bus/train(transit).

Near Field Communication, also known as NFC, is a subset of RFID. NFC operates at a frequency of 13.56MHz and transfers data at a rate of 106k-424k bits per second (Butterfield & Szymanski, 2018). NFC capable devices can be switched to emulate passive or active states, such as smartphones to read and write to each other.  To standardize the flow of information from device to device, NXP Semiconductors, Sony, Nokia and Philips have joined together to create protocols that are found under the NFC Forum name (Vanderkay, 2004)Currently, NFC Forum has 5 different specifications for tags, each unique for their own purposes. For this project, the Type 4 Forum Tag (ISO/IEC 14443) specification will be used to communicate between the hardware and an Android mobile device which. This involves 4 parts: physical characteristics, radio frequency power and signal interface, and initialization and anti-collision (Sabella, 2016). The Type 4 tag was selected due to the ability to hold large amounts of data in NFC Data Exchange Format (NDEF). This will be furthered explained in conjunction with Application Protocol Data Unit(APDU) commands specified in ISO/IEC 7816-4 (Organization, security and commands for interchange)  to show how one device can access data on another device .

Modern day smartphones, capable of doing many tasks, have been very convenient for everyday users. With the ability to perform host card emulation on these Android devices, contactless payment and other data transactions are made possible through NFC from one device to another. This type of technology can be applied to modern day inventory management problems such as the Parts Crib. The Parts Crib currently uses RFID tags that are individually scanned in and out for each incoming student, costing precious time and money. Having a contactless transaction of information with a mobile device will allow the Parts Crib to operate efficiently and effectively without requiring any extra manpower. Since students carry around their phones every day, it is simpler to just tap their phones than to rummage through purses and wallets. This system will allow students to securely transfer their information to the Parts Crib for easy access.

# 3.0 Methodology

## 3.1 Required Resources

Report

/1 Parts/components/materials (500 words)

/1 PCB, case (500 words)

/1 Tools, facilities (500 words)

/1 Shipping, duty, taxes (250 words)

/1 Working time versus lead time (250 words)

### 3.1.1 Parts, Components, Materials

The NFC System Management system revolves around a hardware device that will sense an incoming device, scan NDEF messages containing data via NFC and display the status of the device. The internal hardware component has 4 main devices with supporting electronics parts to deliver the functionality.

The first main component is a Raspberry Pi 3 B that revolves around a system on a chip and has GPIO pins for power and data transfer. The device’s main purpose is to gather data from the sensors and display data to the effectors. To manage and handle the data, it will run scripts to interpret the data and send it off to a database server for further processing.

The second device is the Adafruit PN532 NFC Controller that is capable of many functions but will mainly act as an NFC reader to get access to the NDEF messages sent by an Android mobile device. For the device to power up, it requires a connection to the 3.3V and ground lines from the Raspberry Pi. To communicate with the Raspberry Pi, it needs to be wired in the I2C configuration which requires connections to the Serial Data Line (SDA) and a Serial Clock Line (SCL) line.

The third device is the Adafruit HD44780 16x2 LCD Display that will be used to output messages to the user based on their interaction with our system. After completing a transaction through the device above the output will either display a success or instructions/error code based on the action. The device requires a 5V and ground connection to the Raspberry Pi. To communicate to the Pi using I2C a PCF8574P IC I/O expander chip is used to communicate to the device, this requires 3.3V and ground from the Pi as well as connections to the SDA and SCL lines. To control the contrast and backlight of the LCD there are two 10k ohm potentiometers wired to the LCD as well as a 10k and 4k7 ohm resistor for better control over the lighting options. This setup also provides protection to the LCD so less voltage is being input to the device.

The fourth device is the Adafruit VCNL 4010 proximity sensor which will be used to for user input to the device. The proximity sensor will be set up in such a way so that when the user puts their phone near the device, it will brighten the LCD screen. The VCNL 4010 has an SCL and SDA pin what are used to transfer data. The values the 4010 returns are unit less but need to be converted. The closer you get to the sensor, the higher the proximity value and the lower the ambiance. The sensor’s IR led operates from 3.3 to 5 volts and is not very large as it is smaller than a quarter.

### 3.1.2 PCB and Enclosure Design

When the hardware components have been wired and tested on a breadboard, a printed circuit board (PCB) will be designed in Fritzing with the Adafruit library. It will be designed so that it can be a connecting point for all the main devices and its supporting components. It will also be designed for modularity with troubleshooting in mind so that nonfunctional components can be easily accessed and replaced.

The PCB design will contain holes for female header pins to be soldered on. One 2x20 40-pin female header will be used to connect the PCB to the raspberry pi’s GPIO pins. Also, a 6-pin female header, 9-pin female header, and a 16-pin header will be used for a proximity sensor, an NFC reader and an LCD display respectively. Each header that is dedicated to each device will have lines providing power and I2C communication running from the 40-pin header.  To route these lines, vias will be created so that the lines will not overlap, allowing them to be placed both on top and bottom of the PCB. Any other supporting electrical components such as resistors and transistors will have holes dedicated to them as well. For the PCB to be completely secure and held in place, 4 mounting holes around 2.5-3mm in diameter will be put in so that screws can threaded through to hold the PCB to the nylon standoffs that will be put under it. When the PCB is finished in its designed phase, the files will be sent off to a local prototype lab at Humber College to be printed.

When the PCB is being sent off to be printed, the design files will then be imported into AutoCAD and Inventor to design the case to surround the internal hardware. The external design will be boxy in shape similar to that of a rectangular prism.

The base of the enclosure will be based on the dimensions of the Raspberry Pi. Holes with standoffs will be created so that the Raspberry Pi does not rest on the soldered joints but rather on elevated pads. This will prevent the device from overheating and possibly melting the plastic underneath it. The holes will be countersunk so that the screws to mount the Raspberry Pi to the case will be not exposed.

The sidewalls of the device will have holes created to give access to the Raspberry Pi’s micro USB for power, HDMI, USB ports, Ethernet port, and audio port. Having access to these ports allows easy access for future development and mainly for troubleshooting the device. Extra holes will be put in place to allow the Raspberry Pi’s LED and the NFC reader’s LED to check on the status of other devices.

The removable top-lid of the device will be designed to have holes for the LCD Screen and a very small hole for the proximity sensor. 2.5-3mm mounting holes will be put in place for each device so that they do not shift around during transportation or during a transaction.

### 3.1.3 Tools and Facilities

The initial setup of our final product will require the integration of all our devices that we will build on a breadboard and interact with through a computer to set up necessary files to run our program. This will be mostly done in the J232 Lab room at Humber College. Use of the digital multimeter in the room is important for ensuring the right voltage levels are present in specific parts of the build. Schematics, breadboard designs, and PCB design will all be created on Fritzing before each phase is actually made.

With past experience it is noted important in PCB design to ensure leads at each connector is facing a certain way so the solder matches the same side. This makes soldering for each connector onto the board a lot easier to do.

After completing the tests required for our devices to all work on the same platform together, the custom PCB (Printed Circuit Board) and enclosure will be designed and created through the facilities at Humber College.

All of our past custom PCBs were created after submitting our designs to the Prototype Lab at Humber College. Our past enclosures were split between submitting a design for laser-cutting and 3D printing at the same location. Through comparing the pros and cons of both methods we plan on using the same methods to have our final product’s PCB and enclosure created at the Prototype Lab with the enclosure method leaning towards a heavier laser-cut design. Preference is over a full laser-cut enclosure however through 3D printing the portions of the enclosure requiring mounted screws can be created with a better design.

After the PCB is created the assembly will take place in the J232 Lab room. This facility provides soldering stations with fume extractors to provide user safety. This room will be an important factor towards the assembly step of the PCB as it is an easy access point to being able to solder our PCB and in safe work conditions. A long with the facility the use of pliers and wire cutters are required to prepare fine wire, shape pieces such as connectors, and trim the smaller pieces that will be soldered onto the PCB (example: resistors). Safety goggles will be needed for any worker near the solder work to prevent any exposure of chemicals to the eye, and the fume extractor will lower the risk of inhaling of any chemicals.

Through past solder work, the digital multimeter was found to be important in this step as well as it helps ensure connections are made on the PCB as well as be able to test voltage levels for the devices when it is assembled.

After this phase of testing is completed we are able to attach our PCB onto the Raspberry Pi to ensure the devices work and can interact with our microprocessor. The testing here will be similar to the breadboard testing however we will also confirm the size dimensions with the product as a whole and determine the finalization for sizes and designs towards the enclosure.

Lastly enclosure assembly will require some adhesive for some portions and the use of a screwdriver to add the necessary screws and nuts to bring the pieces of the enclosure together with the final product fastened to it.

### 3.1.4 Shipping, duty, taxes

The main components, PN532 NFC, Raspberry Pi, HDF8574P LCD screen, and the VCNL 4010 proximity sensor, were all obtained mainly from amazon or directly from the manufacturer. The majority of these places do apply taxes but do not charge for shipping or duty. The individual costs of the parts are the following: Raspberry pi was $45.75, the HD44780 LCD screen was $9.95, the VCNL 4010 proximity sensor was $7.50 and the PN532 NFC reader was $71.42. All these combined was $134.62 CAD. With taxes, this brings to total to $152.12 CAD which is 17.50 in taxes. The other smaller components include: the nylon stand-off kit, which was $13.99, the PCF8574P Remote 8-bit I/O expander IC which was $9.95, a 10K resistor which was $0.15, and a 4k resistor which was also $0.15. With these components added it then brings the grand total to $161.86 and $182.90 after tax. This means that the taxes we will be paying will be $21.04 for all our materials. The shipping states that the parts should arrive in 5-7 business days however, in the past, orders were delayed and adjustments had to be made to the project plan in order to ensure that the project would be done on time. For example, when Colin ordered his blue pill, it got delayed by a week and he had to make adjustments to his schedule to ensure he finished it on time. We are prepared to make changes to our schedule in case unforeseen events like delayed shipments take place.

### 3.1.5 Time expenditure

Working time for our project will mostly be done outside of the allocated lab time. The exception to this will be for soldering the PCB and printing and assembling the case. We plan to assemble most of our hardware outside of the lab as we have allocated out workload and set dates to follow. This makes it easier and more efficient as we can get more work done in a shorter period of time since all of the group members will be contributing to different parts of the project at the same time. We plan to use most of the lab time as a meet up to troubleshoot any issues and to make plans for future steps. We will be spending out working time polishing the mobile application, developing the web application, and creating the hardware for the application. The lead time will consist of brainstorming, troubleshooting, and discussing future plans and any possible changes to schedule or overall direction. Working time will also be used to test and debug any issues that might occur during the development of this project. Since we do most of the development outside of lab time, we communicate any issues we have via a group chat and assist each other when possible. Following this work ethic is what we believe to be the best possible way to approach this project in terms of working time and lead time.

## 3.2 Development Platform

The mobile application will be developed using java with Android Studio version 3.5 with API 21. The app will also coexist and communicate with the web server that handles requests through PHP to an SQL database.

### 3.2.1 Mobile Application

The app is currently working as intended and communicates with the server with no issues. The only thing that must be completed on the application is the communication with the web application and polishing the GUI. The home screen on the app (Figure 4) is going to include a recycler view were the parts crib employees can send notifications to users via the web application. The GUI also needs some work as it was made quick and with low priority since we needed the major functions of the app to work. Going further into the semester Colin will be working to polish the GUI to make it more appealing and easier to follow since that was one of the goals. Since the major functions of the app are now operational, polishing the GUI should not be too time consuming since it just requires fixing the XML files. The mobile application is due March the 14th 2020 and there should be no issues getting it done by that date.

The mobile application was made in android studio using API level 21. The current stable version on the application works as intended and just needs to be polished to be more aesthetically pleasing. The app was developed with the intention to make it easy and straight forward to use. The project was divided into three parts; the GUI, the database, and the back end. These parts were allocated to the three group members and work was completed on time. The app has a total of 11 screens and was designed to be easy to navigate. The main activity that will load after the splash screen is the login screen.

Figure 1. Main Activity

The main activity (Figure 1) was made using two custom buttons and a simple image placed at the top of the screen. The two buttons lead to the two corresponding activities.

Figure 2. Create Account Screen

Figure 2. Create Account Screen

Figure 2. Create Account Screen

The create account (Figure 2) activity was made with custom input boxes and a custom button. This screen was made to mirror other create account pages or screen. In order for an account to be created, all of the fields must have valid information in them. If the user tries to make an account while having no information entered, it will alert the user that they need to fill in the information. This was crucial when creating this screen since the database would be full of incomplete information.

Figure 3. Login Screen

The login screen (Figure 3) was made to also be simple and straight forward. This screen is also where the admin can login once the web application is completed. This screen was made with two custom input boxes, a custom button, a remember me check box, that saves the users login information on the phone’s memory, and an admin login.

Figure 4. Home Screen

Figure 5. Navigation Drawer

The home activity (Figure 4) was made by using a navigation drawer (Figure 5) that lead to the other main activities. This screen will also have notifications from the parts crib. The notifications feature will be implemented when the web application is completed.

Figure 6. Items Category Screen

The items category (Figure 6) uses a list view that pulls data from the SQL database to display the current items available at the parts crib that the user can borrow from the parts crib. All of screens from the navigation drawer also have a back button on the top left to make navigation easier. This was done by making separate processes in the manifest file.

Figure 7. Items Name Screen

The items name screen (Figure 7) is similar to the item’s category screen (Figure 6) as they both use a list view and pull data from the SQL data base to display to the user.

Figure 8. Item Description Screen

The item description screen (Figure 8) can is accessed from pressing on the desired item and it pulls the corresponding information from the SQL server to display to the user.

Figure 9. Cart Screen

The cart screen (Figure 9) is another list view that gives the user an overview of what they are currently planning to request. The cart screen can be accessed from the navigation drawer (Figure 5) or the floating action button on the home screen (Figure 4). Once the user presses the request button, the parts crib will receive a notification the web application to prepare the order.

Figure 10. Orders Screen

The orders screen (Figure 10) displays all the previous orders that the user has made. This is just a list view that pulls order details from the SQL server. Pressing on the order will display what was in the order and if it has been returned or not.

Figure 11. Data Visualization

The data in our app mainly flows from the client’s mobile device to the web server. The web server also communicated with the database in order to pull information to display to the user. For example, when the user wants to view their older orders, the data is sent from the user and then the server pulls the information from the database to display to the user’s mobile device. The user’s mobile device is also going to communicate with the NFC reader at the parts crib. Once the user is ready to pick up or return their order, they tap a button in the app to get into an NFC state to transfer transaction ID to the NFC reader. This transaction ID will be used to look up the order in the database and update the status of the order as picked up/returned.

/1 Hardware present?

/1 Memo by student A + How did you make your Mobile Application? (500 words)

/1 Login activity

/1 Data visualization activity

/1 Action control activity

Include screenshots such as Figure 1. Testing. Progress.

### 3.2.2 Image/firmware

The Raspberry Pi uses the current up-to-date image of Raspbian (Buster) operating system found at their main website. The image was flashed onto the 16GB SanDisk SD card that is used with the Pi. The process was done using an application called ‘balenaEtcher’, this application is used for flashing operating systems onto SD cards and USB drives.

The firmware used for our product was compiled through three different methods used by each member to initially implement their sensor/effector from our hardware projects last semester. Each device was setup to communicate with the Pi via I2C.

The VCNL4010 Proximity Sensor is written in C++ with the source code provided by Adafruit repository for the sensor. The main code used reads the proximity value given by the sensor every 3ms, when the value surpasses 2300 units the program will initiate the set of code for the NFC portion of the task.

The NFC reader PN532 utilizes example code provided by nfc-tools.org as well as their wiringPi libraries and is written in C. Upon activation the NFC sensor begins a process by first detecting the nearby device then selecting the application ID to receive an NDEF (NFC Data Exchange Format) message that contains the transaction ID of the client from their mobile device. If the device does not have the corresponding application ID the process will terminate. When a NDEF message is successfully the program will initiate the script for the LCD with the information given through a system call.

Using a guide provided by Rototron.info as well as the libraries provided by Adafruit the HD44780 LCD screen is able to display strings written to its respective address from the NFC function. The script used to display strings on the LCD was written in Python.

Our goal is to achieve communication between our device and the PHP script that handles our database. We plan to do this over HTML using GET and POST to update the server and complete the transaction process at the Parts Crib when the client has tapped their mobile device. Documentation of the resources used to achieve this will be provided at the repository for reference.

### 3.2.3 Connectivity

The user has the two different methods should the microprocessor need to be accessed for any adjustment or maintenance.

The Raspberry Pi when setup and in Wi-Fi range can be accessed remotely from another terminal using RealVNC Server/Client. Provided in the wpa_supplicant.conf file on the Pi are the credentials for access to the Humber College Wi-Fi under a student account. With this enabled any user logged into the RealVNC team on the client application can access the device remotely. Internet connection can also be established by connecting an Ethernet cable between the Pi and any networking device that has connection to the internet.

If internet connection cannot be established the Pi can also be communicated to by connecting the Pi to another device via Ethernet and using the network application Bonjour or any similar functioning tool. Bonjour is the application selected by our team and it is used to help setup a network between connected devices. This allows the connected device to assign an IP to the Pi so that it can be remotely connected to.

If neither of these options are a viable option for the scenario then peripherals can be connected directly to the Pi and interacted with.

3.2.2 Image/firmware

Status

/1 Hardware present?

/1 Memo by student B + How did you make your Image/firmware? (500 words)

/1 Code can be run via serial or remote desktop

/1 Wireless connectivity

/1 Sensor/effector code on repository

### 3.2.4 Breadboard/Independent PCBs

The hardware seen in this system were all designed in the previous semester. The LCD, the proximity/luminosity sensor and the NFC Controller were all carefully tested using the I2C interface before the integration on two separate breadboards. Due to the large amounts of components and required space, the LCD needed a dedicated breadboard while the rest shared a single breadboard.

The LCD, in general, requires a lot of wire when it used in a parallel interface. Though it is almost impossible to run all those connections to a Raspberry Pi GPIO, the LCD uses a PCF8574 IC to use the I2C interface to convert the parallel interface into a serial one. To run in an I2C interface, the PCF8574 IC needs 3V, SDA, SCL and ground to be connected from the Raspberry Pi while the remaining connections are meant to be used to control the LCD. Using the potentiometers with the 4k7 and 10k resistors tied to them allows control over the backlight of the display and the light of the text (the output) during the testing phase of the device. The potentiometer will later be replaced with fixed value resistors to keep the device more compact.

Figure 12. LCD Schematic

The VCNL 4010 proximity/luminosity sensor, previously tested on an STM32 V2 link Blue Pill (an Arduino alternative that uses its IDE), also uses the I2C interface. It just needs to be daisy chained off of the LCD’s connections which are the: 3V, SDA, SCL, and ground. To connect to the device through I2C its address is 0x13.

Figure 13. Proximity/Luminosity Sensor

The PN532 controller also uses the I2C interface which is daisy-chained off of the VCNL 4010 using the 3V, SDA, SCL and ground connections. The hardware itself requires a jumper on two sets of pins. Located on the board is labeled SE0 and SE1 where they have 3 pins each. There are several interfaces that can be selected on this NFC controller which include: I2C, UART, and SPI. In this case, I2C is being used so SE0 has to have the jumpers on the ON position while SE1 is switched off. To communicate with the device, it uses a I2C address of 0x24.

Figure 14. NFC Controller Schematic

Even though the device as a whole currently is operational, there are still some technical difficulties that need to be addressed. The PN532 is the core of the project and is currently experiencing some issues. It is currently reading corrupted data when there is an interruption with an ongoing data transaction. When it is unable to complete a data transaction, it will begin to corrupt the next incoming data transaction as well. Upon further inspection, there is a reset pin on the device. The reset pin can be attached to a GPIO pin on the Raspberry PI to control when to reset the device, but more reading has to be done on the device before any changes are made. Also, the group had also noticed that the backlight did not need to be on all the time especially when no one is using it because it was a waste of energy. The idea of a transistor was brought up to control the backlight and will be later implemented into the design of the project.

Figure 15. Schematic for all devices wired together

Figure 16. Breadboard Layout. VCNL4010(Top), LCD(Bottom), PN532(Right)

Status

/1 Hardware present?

/1 Memo by student C + How did you make your hardware? (500 words)

/1 Sensor/effector 1 functional

/1 Sensor/effector 2 functional

/1 Sensor/effector 3 functional

The initial schematic design, Figure 2, based on datasheets (Bosch Sensortec, 2019) led to a breadboard layout Figure 3 that was realized Figure 4.

How did you build your Prototype: Breadboard?

Then a PCB was designed, Figure 5, and populated (Figure 6). Bill of Materials, Case, Time commitment. Testing. Progress.

### 3.2.5 Printed Circuit Board

This section documents the PCB created for our product. Outlined will be the process as well as figures to explain how the prototype PCB was created.

The first step in our process was taking a look over each of the independent PCBs from our previous semester and removing the components that would not be required for the combined prototype (LEDs, potentiometers). After close inspection, we were left with our two sensors, the LCD display, the IC chip, and a few resistors to handle the lighting for the LCD. With the initial schematic for the LCD, the back and foreground lights were adjustable with potentiometers but were later replaced with resistors and a 2N4124 NPN transistor that is controlled by a GPIO pin, so that the backlight can be controlled by software. The last step of the planning process was deciding the size of the PCB that would affect the enclosure and the prototype as a whole. The size was determined by the largest component the NFC sensor with the LCD display and proximity sensor sitting above the NFC sensor and the Raspberry Pi underneath to seat the PCB itself.

After the initial preparation, the PCB design was later designed in Fritzing. The wiring necessary for each component was designed to lead into connectors that would have pin connections for only the pins required for each component. Due to the size of the NFC sensor the decision was made to have a wired connection from the connectors to the LCD display and proximity sensor, then the two components are to be seated within the enclosure. This decision was made to prevent the dimensions of the enclosure from being too large.

Figure 17. PCB Design

Figure 18 Top of PCB

Figure 19  Bottom of PCB

The file that was designed in Fritzing was then submitted to the Humber Prototype lab for the PCB to be developed. The soldering of the connectors, resistors as well as completing the wires through the VIAs onto the PCB took less than two hours to put together.

Figure 20 Top of PCB after soldering

Figure 21. Bottom of PCB after soldering

The testing of the PCB was conducted with a multi-meter to ensure all the copper lines were connected, correct voltages were being delivered and to ensure there were no short circuits. With the success of this testing, the next step was to test the PCB with the main components plugged into the connectors. At this stage, there was one issue that required troubleshooting. The IC chip used for our setup was running at a high temperature. To resolve this issue, a wire was ran from the grounding point near the IC directly to the ground of the female header that is connected to the Raspberry Pi. Further testing showed that all devices had valid I2C addresses shown to the Pi and all components could communicate through our software.

Figure 22. Completed PCB with connected components

Demo

/1 Hardware present?

/1 PCB Complete and correct

/1 PCB Soldered wire visible but trim, no holes or vacancies

/1 PCB Tested with multimeter

/1 PCB Powered up

How did you build your Prototype: PCB?

### 3.2.5 Enclosure

Demo

/1 Hardware present?

/1 Case encloses development platform and custom PCB.

/1 Appropriate parts securely attached.

/1 Appropriate parts accessible.

/1 Design file in repository, photo in report.

How did you build your Prototype: Case?

The main base of the enclosure that mounts all the hardware components was 3D printed using ABS plastic while the top lid portion was laser cut using acrylic. The length, width and height (mm) are 158.1, 87.5, 61.0 respectively.  The enclosure was designed to be as compact as possible but still leave enough room to insert screws, manage wires and mount devices on top of each other.

The process to develop the device initially required careful measurements to retain functionalities such as when a phone is being sensed or scanned from a distance. After the measurements were taken, the work was then done on the computer to create a 2D layout of the enclosure. The Fritzing PCB diagram that was created before was exported as a PDF and then modified in AutoCad. Here the measurements that were taken, were drawn and the holes from the PDF were redrawn to a specific size to accommodate heat set threaded inserts. The measurements for the holes were 3mm for the 2.5mm inserts and 4mm for the 3mm inserts. The CAD file was then exported into AutoDesk Inventor where base and walls of the enclosure were extruded. 2 areas were then cut out to expose the I/O ports and micro USB port to make them more accessible. After, standoffs were created for the LCD(4mm), the proximity sensor(3mm) and the top lid(4mm). These standoffs were design with angles less than 45 degrees from the sidewalls. Anything over 45 degrees will cause the 3D printer to start printer supports for the component when it does not have enough time to cool down and hold its shape. When the standoffs were completed, the file was then exported as an STL and sent off to the Prototype Lab to be printed. It took approximately 4-5 hours to be printed using a Stratasys printer and was picked up the next day.

Heat-set threaded inserts were then placed into their required holes using a soldering iron with a temperature of 750 degrees Celsius. When the inserts were almost seated, an object with a straight flat surface was used to push the insert flush to the plastic so that screws can be threaded straight. Once all the inserts were in, the hardware components were then briefly mounted and were only held with several screws to check the fitment of the enclosure.

When everything looked adequate, a lid was then quickly designed in Inventor with the base of the enclosure as reference. The lid was designed to have (4) 3.5mm holes for the 3mmx8mm screws to secure the lid to the standoffs that were created on the walls of the enclosure. It also has a cut out for the LCD to protrude and a small square for the proximity sensor so that it does not fully detect the acrylic that will be laid on top of it. Once completed, only the sketch was exported to a DXF file and sent to the Prototype Lab for laser cutting on a piece of clear acrylic that was 3mm thick. The laser cutting was done in approximately 10 minutes and the enclosure with the components were taken apart for reassembly and wire management.

The assembly of the enclosure included the following materials as seen in Figure ..

| Hardware | Materials | Notes |
| --- | --- | --- |
| Raspberry Pi 3 B+ | 4 M2.5*12+6mm standoffs 3 M2.5 nuts | The standoffs are attached to the Raspberry Pi with the nuts. With the remaining threads extending out from the nuts, these are then threaded into the heat set inserts of the enclosure. |
| PCB | 2x M3*10+6mm standoffs 2x M3*6+6mm standoffs 2x M3 nuts 3 M3x15+6mm standoffs 3 M3 screws 4 M2.5+6mm screws | One M3*10 and one M3*6 are threaded together to create two M3*16 standoffs. These standoffs are attached to the PCB with the M3 nuts so that the NFC reader can later be mounted. The M*15 standoffs are threaded into the heat set inserts of the enclosure. The PCB is then mounted onto the Raspberry Pi and the standoffs that are mounted to enclosure. The PCB is then attached with 4 M2.5 screws to keep them in place. |
| PN532(NFC Reader) | 2x M3*20+6mm standoffs 2x M3*12+6mm standoffs 4x M3+6mm screws | One M3*20 and one M3*12 are threaded together to create two M3*32 standoffs. These are then threaded into the heat set inserts of the enclosure. The NFC reader is then mounted to the 2 standoffs on the PCB and the 2 standoffs mounted to the enclosure |
| VNCL 4010 Proximity Sensor | 2 M2.5+6mm screws |  |

## 3.3 Integration

Demo

/1 Hardware present?

/1 Data sent by hardware

/1 Data retrieved by mobile application

/1 Action initiated by mobile application

/1 Action recieved by hardware

Report

/1 Enterprise wireless connectivity (250)

/1 Database configuration (250 words)

/1 Security considerations (500 words)

/1 Unit testing (900 words)

/1 Production testing (100 words)

### 3.3.1 Enterprise Wireless Connectivity

How did you make a Database accessible by both your Prototype and Mobile Application?

### 3.3.2 Database Configuration

### 3.3.3 Security

### 3.3.4 Testing

Unit testing and Production testing.

# 4.0 Results and Discussions

Is your prototype perfect? What did you learn?

# 5.0 Conclusions

If you were making 1000 of these.

Report

/1 Hardware present?

/1 Checklist truthful

/1 Valid Comments

/1 Results and Discussion (500 words)

/1 Conclusion

# 6.0 References

Bosch Sensortec. (2019, July). BME680 - Datasheet. Retrieved from Robert Bosch GmbH: https://ae-bst.resource.bosch.com/media/_tech/media/datasheets/BST-BME680-DS001.pdf

Butterfield, A., & Szymanski, J. (2018). In A Dictionary of Electronics and Electrical Engineering. Oxford University Press.

Hasing Security. (2019, June 5). Retrieved from Crack Station: https://crackstation.net/hashing-security.htm

OACETT. (2017, March). I need to Complete a Technology Report. Retrieved from The Ontario Association of Certified Engineering Technicians and Technologists: https://www.oacett.org/Membership/Technology-Report-and-Seminar

Sabella, R. (2016, April 11). NFC for dummies. Retrieved from dummies: https://www.dummies.com/consumer-electronics/nfc-standard-isoiec-14443/

Vanderkay, J. (2004, March 18). Nokia, Philips And Sony Establish The Near Field Communication (NFC) Forum. Retrieved from NFC Forum: https://nfc-forum.org/nokia-philips-and-sony-establish-the-near-field-communication-nfc-forum/

# 7.0 Appendix

## 7.1 Firmware code

Demo

/1 Hardware present?

/3 Code runs concurrently for all sensors/effectors

/1 Project repository contains integrated code

Status

/1 Memo including updates

/1 Financial update

/1 Progress update

/1 Modified Code Files in Appendix

/1 Link to Complete Code in Repository

## 7.1.1 main.c File Reference

Main Program that will run the VNCL 4010, PN532 and LCD.

#include <stdio.h>

#include <stdlib.h>

#include <signal.h>

#include <string.h>

#include <unistd.h>

#include <nfc/nfc.h>

#include "Presence.h"

#include "NDEFData.h"

### Functions

- int main (void)

Function used to initialize Presence and setup the NFC reader.

### Detailed Description

Main Program that will run the VNCL 4010, PN532 and LCD.

##### Author

Robert Dinh

##### Date

01MAR2020

Definition in file main.c.

### Function Documentation

#### int main (void )

Function used to initialize Presence and setup the NFC reader.

The fuction sets up the VNCL4010 and PN532 using the Wiring Pi library through I2C. It then sets up the registers of the devices so that data can be retrieved. When a presence is detected by the VNCL4010 it triggers the NFC reader into a read state to to grab an NDEF encoded message from a mobile device.

Definition at line 27 of file main.c.

## main.c

00001

00008 #include <stdio.h>

00009 #include <stdlib.h>

00010 #include <signal.h>

00011 #include <string.h>

00012 #include <unistd.h>

00013 #include <nfc/nfc.h>

00014 #include "Presence.h"

00015 #include "NDEFData.h"

00016

00017

00027 int main (void)

00028 {

00029     initNFC();

00030     int fd = initPresence();

00031

00032     int present = 0;

00033

00034     fprintf(stdout,"%d \n",fd);

00035     present = detectPresence(fd);

00036     if (present)

00037         getNdef();

00038

00039     stopFunction(2);

00040     return 0;

00041 }

## 7.1.2 NDEFData.c File Reference

Various Functions to read an NDEF message from a mobile device.

#include <stdlib.h>

#include <string.h>

#include <unistd.h>

#include <signal.h>

#include <nfc/nfc.h>

### Functions

- void stopFunction (int sig)

Function used to stop the NFC reader in the case of a System Interrupt.

- void initNFC (void)

Function used to initialize the PN532 NFC Reader.

- int CardTransmit (nfc_device *pnd, uint8_t *capdu, size_t capdulen, uint8_t *rapdu, size_t *rapdulen)

Function used to send APDU commands.

- void getNdef (void)

Function used to initialize Presence.

### Variables

- nfc_device * pnd

- nfc_context * context

### Detailed Description

Various Functions to read an NDEF message from a mobile device.

##### Author

Robert Dinh

##### Date

01MAR2020

The following does contain non-original code but was modified to suit certain needs. Original code was developed by NFC - TOOLS. http://nfc-tools.org/index.php/Libnfc:APDU_example

Definition in file NDEFData.c.

### Function Documentation

#### int CardTransmit (nfc_device *  pnd, uint8_t *  capdu, size_t  capdulen, uint8_t *  rapdu, size_t *  rapdulen)

Function used to send APDU commands.

Definition at line 70 of file NDEFData.c.

#### void getNdef (void )

Function used to initialize Presence.

Definition at line 106 of file NDEFData.c.

#### void initNFC (void )

Function used to initialize the PN532 NFC Reader.

Definition at line 42 of file NDEFData.c.

#### void stopFunction (int  sig)

Function used to stop the NFC reader in the case of a System Interrupt.

Definition at line 28 of file NDEFData.c.

## NDEFData.c

00001

00013 #include <stdlib.h>

00014 #include <string.h>

00015 #include <unistd.h>

00016 #include <signal.h>

00017 #include <nfc/nfc.h>

00018

00019 //NFC Variables

00020  nfc_device *pnd;

00021  nfc_context *context;

00022

00023

00028 void stopFunction(int sig)

00029 {

00030

00031     printf("Stopping now.\n");

00032     nfc_close(pnd);

00033     nfc_exit(context);

00034     exit(EXIT_SUCCESS);

00035

00036 }

00037

00042 void initNFC(void)

00043 {

00044     signal(SIGINT,stopFunction);

00045     nfc_init(&context);

00046

00047     if (context == NULL) {

00048         printf("Unable to init libnfc (malloc)\n");

00049         exit(EXIT_FAILURE);

00050     }

00051

00052     pnd = nfc_open(context, NULL);

00053

00054     if (pnd == NULL) {

00055         printf("ERROR: %s", "Unable to open NFC device.");

00056         exit(EXIT_FAILURE);

00057     }

00058

00059     if (nfc_initiator_init(pnd) < 0) {

00060         nfc_perror(pnd, "nfc_initiator_init");

00061         exit(EXIT_FAILURE);

00062     }

00063

00064 }

00065

00070 int CardTransmit(nfc_device *pnd, uint8_t * capdu, size_t capdulen, uint8_t * rapdu, size_t * rapdulen)

00071 {

00072     int res;

00073     size_t  szPos;

00074

00075     printf("=> ");

00076     for (szPos = 0; szPos < capdulen; szPos++)

00077     {

00078         printf("%02x ", capdu[szPos]);

00079     }

00080     printf("\n");

00081

00082     if ((res = nfc_initiator_transceive_bytes(pnd, capdu, capdulen, rapdu, *rapdulen, 500)) < 0)

00083     {

00084         return -1;

00085     }

00086     else

00087     {

00088         *rapdulen = (size_t) res;

00089         printf("<= ");

00090

00091         for (szPos = 0; szPos < *rapdulen; szPos++)

00092         {

00093           printf("%02x ", rapdu[szPos]);

00094         }

00095

00096     printf("\n");

00097     return 0;

00098     }

00099 }

00100

00101

00106 void getNdef(void)

00107 {

00108     nfc_target nt;

00109

00110

00111     const nfc_modulation nmMifare = {

00112     .nmt = NMT_ISO14443A,

00113     .nbr = NBR_106,

00114     };

00115

00116     printf("Polling for target...\n");

00117

00118     while (nfc_initiator_select_passive_target(pnd, nmMifare, NULL, 0, &nt) <= 0);

00119     printf("Target detected!\n");

00120

00121     uint8_t capdu[264];

00122     size_t capdulen;

00123     uint8_t rapdu[264];

00124     size_t rapdulen;

00125

00126

00127     // Select application

00128     memcpy(capdu, "\x00\xA4\x04\x00\x07\xF0\x39\x41\x48\x14\x81\x00\x00", 13);

00129     capdulen=13;

00130     rapdulen=sizeof(rapdu);

00131     if (CardTransmit(pnd, capdu, capdulen, rapdu, &rapdulen) < 0)

00132         exit(EXIT_FAILURE);

00133     if (rapdulen < 2 || rapdu[rapdulen-2] != 0x90 || rapdu[rapdulen-1] != 0x00)

00134         exit(EXIT_FAILURE);

00135

00136     printf("Application selected!\n");

00137

00138     // Select Capability Container

00139     memcpy(capdu, "\x00\xa4\x00\x0c\x02\xe1\x03", 7);

00140     capdulen=7;

00141     rapdulen=sizeof(rapdu);

00142     if (CardTransmit(pnd, capdu, capdulen, rapdu, &rapdulen) < 0)

00143         exit(EXIT_FAILURE);

00144     if (rapdulen < 2 || rapdu[rapdulen-2] != 0x90 || rapdu[rapdulen-1] != 0x00) {

00145         capdu[3]='\x00'; // Maybe an older Tag4 ?

00146     if (CardTransmit(pnd, capdu, capdulen, rapdu, &rapdulen) < 0)

00147         exit(EXIT_FAILURE);

00148     }

00149

00150     printf("Capability Container selected!\n");

00151

00152     // Read Capability Container

00153     memcpy(capdu, "\x00\xb0\x00\x00\x0f", 5);

00154     capdulen=5;

00155     rapdulen=sizeof(rapdu);

00156     if (CardTransmit(pnd, capdu, capdulen, rapdu, &rapdulen) < 0)

00157         exit(EXIT_FAILURE);

00158     if (rapdulen < 2 || rapdu[rapdulen-2] != 0x90 || rapdu[rapdulen-1] != 0x00)

00159         exit(EXIT_FAILURE);

00160

00161     printf("Capability Container header:\n");

00162

00163     size_t  szPos;

00164     for (szPos = 0; szPos < rapdulen-2; szPos++)

00165     {

00166         printf("%02x ", rapdu[szPos]);

00167     }

00168     printf("\n");

00169

00170     // NDEF SELECT

00171     memcpy(capdu, "\x00\xA4\x00\x0C\x02\xE1\x04", 7);

00172     capdulen=7;

00173     rapdulen=sizeof(rapdu);

00174     if (CardTransmit(pnd, capdu, capdulen, rapdu, &rapdulen) < 0)

00175         exit(EXIT_FAILURE);

00176     if (rapdulen < 2 || rapdu[rapdulen-2] != 0x90 || rapdu[rapdulen-1] != 0x00)

00177         exit(EXIT_FAILURE);

00178

00179     printf("NDEF SELECTED!\n");

00180

00181     // NDEF Read Binary

00182     memcpy(capdu, "\x00\xb0\x00\x00\x02", 5);

00183     capdulen=5;

00184     rapdulen=sizeof(rapdu);

00185     if (CardTransmit(pnd, capdu, capdulen, rapdu, &rapdulen) < 0)

00186         exit(EXIT_FAILURE);

00187     if (rapdulen < 2 || rapdu[rapdulen-2] != 0x90 || rapdu[rapdulen-1] != 0x00)

00188         exit(EXIT_FAILURE);

00189     printf("NDEF Read Binary NLEN!\n");

00190

00191     //READING NDEF DATA

00192     memcpy(capdu, "\x00\xb0\x00\x00\x0f", 5);

00193     capdulen=5;

00194     rapdulen=sizeof(rapdu);

00195     if (CardTransmit(pnd, capdu, capdulen, rapdu, &rapdulen) < 0)

00196         exit(EXIT_FAILURE);

00197     if (rapdulen < 2 || rapdu[rapdulen-2] != 0x90 || rapdu[rapdulen-1] != 0x00)

00198         exit(EXIT_FAILURE);

00199

00200     printf("NDEF DATA ! \n\n");

00201     size_t  szPos2;

00202     char ndefMsg[100] = "";

00203     char chr[1] = "\0";

00204     int counter = 0;

00205

00206     for (szPos2 = 8; szPos2 < rapdulen-2; szPos2++) {

00207         ndefMsg[counter]=(char)rapdu[szPos2];

00208         counter++;

00209     }

00210

00211     printf("%s",ndefMsg);

00212

00213     char cmdS[100] = "python lcd.py Phone Scanned\n";

00214

00215

00216     system(cmdS);

00217     sleep(1);

00218     printf("\nDone...\n");

00219 }

## NDEFData.h File Reference

Function prototypes for NDEFData.

### Detailed Description

Function prototypes for NDEFData.

##### Author

Robert Dinh

##### Date

01MAR2020

Definition in file NDEFData.h.

## NDEFData.h

00001

00008 #ifndef NDEFData_H

00009 #define NDEFData_H

00010

00011

00013 // Function Prototypes

00014 void stopFunction(int sig);

00015 void initNFC(void);

00016 int CardTransmit(nfc_device *pnd, uint8_t * capdu, size_t capdulen, uint8_t * rapdu, size_t * rapdulen);

00017 void getNdef(void);

00019

00020

00021

00022 #endif

## 7.1.3 Presence.c File Reference

Various functions to aid in Presence Detection with the VNCL4010.

#include <stdio.h>

#include <wiringPiI2C.h>

#include <unistd.h>

#include "Presence.h"

### Functions

- int initPresence (void)

Function used to initialize Presence.

- int detectPresence (int fd)

Function used to detect the presence of a person.

- int getProximity (int fd)

Function used get the proximity of the sensor.

- int getLuminosity (int fd)

Function used get the luminosity of the sensor.

### Detailed Description

Various functions to aid in Presence Detection with the VNCL4010.

##### Author

Robert Dinh

##### Date

01MAR2020

Definition in file Presence.c.

### Function Documentation

#### int detectPresence (int  fd)

Function used to detect the presence of a person.

Definition at line 44 of file Presence.c.

#### int getLuminosity (int  fd)

Function used get the luminosity of the sensor.

Definition at line 80 of file Presence.c.

#### int getProximity (int  fd)

Function used get the proximity of the sensor.

Definition at line 65 of file Presence.c.

#### int initPresence (void )

Function used to initialize Presence.

The fuction sets up the VNCL4010 using the Wiring Pi library through I2C. It then sets up the registers of the device so that data can be retrieved.

Definition at line 20 of file Presence.c.

## Presence.c

00001

00008 #include <stdio.h>

00009 #include <wiringPiI2C.h>

00010 #include <unistd.h>

00011 #include "Presence.h"

00012

00013

00020 int initPresence(void)

00021 {

00022     //Setup Wiring Pi thorugh I2C

00023     int fd = wiringPiI2CSetup(0x13);

00024

00025     //Select Command Register

00026     wiringPiI2CWriteReg8(fd,0x80,0xFF);

00027

00028     //Select Prox Register

00029     wiringPiI2CWriteReg8(fd,0x82,0x03);

00030

00031     //Select Current for IR LEDto 200ma

00032     wiringPiI2CWriteReg8(fd,0x83,0x14);

00033

00034     //Select ALS Register

00035     wiringPiI2CWriteReg8(fd,0x84,0x9D);

00036

00037     return fd;

00038 }

00039

00044 int detectPresence(int fd)

00045 {

00046         presenceD pData  = {0};

00047         while (1)

00048         {

00049             pData.proximity = getProximity(fd);

00050             pData.luminosity = getLuminosity(fd);

00051

00052             fprintf(stdout,"Proximity:\t%d\t Luminosity:\t%d\n",pData.proximity,pData.luminosity);

00053             if (pData.proximity > 2300)

00054                 return pData.proximity;

00055

00056             usleep(300000);

00057         }

00058

00059 }

00060

00065 int getProximity(int fd)

00066 {

00067     int prox1= 0;

00068     int prox2= 0;

00069

00070     prox1 = wiringPiI2CReadReg8(fd,0x87) *256; //Upper Byte

00071     prox2 = wiringPiI2CReadReg8(fd,0x88); //Lower Byte

00072

00073     return prox1+prox2;

00074 }

00075

00080 int getLuminosity(int fd)

00081 {

00082     int lux1= 0;

00083     int lux2= 0;

00084

00085     lux1 = wiringPiI2CReadReg8(fd,0x85) *256; //Upper Byte

00086     lux2 = wiringPiI2CReadReg8(fd,0x86); //Lower Byte

00087

00088     return lux1 + lux2;

00089 }

## Presence.h File Reference

Constants, structures, function prototypes for Presence.

#include <wiringPiI2C.h>

### Data Structures

- struct PresenceData

### Typedefs

- typedef struct PresenceData presenceD

### Detailed Description

Constants, structures, function prototypes for Presence.

##### Author

Robert Dinh

##### Date

01MAR2020

Definition in file Presence.h.

## Presence.h

00001

00008 #ifndef PRESENCE_H

00009 #define PRESENCE_H

00010

00011 #include <wiringPiI2C.h>

00012

00013 // Structures

00014 typedef struct PresenceData

00015 {

00016         int proximity;

00017         int luminosity;

00018 }presenceD;

00019

00021 // Function Prototypes

00022 int initPresence(void);

00023 int detectPresence(int fd);

00024 int getProximity(int fd);

00025 int getLuminosity(int fd);

00027

00028

00029

00030 #endif

## 7.1.4 Lcd.py File Reference

from Adafruit_CharLCD import Adafruit_CharLCD

import Adafruit_GPIO.PCF8574 as PCF

import wiringpi as wiringpi

import time

from time import sleep

import sys

#Initialization of the Wiring Pi Library

wiringpi.wiringPiSetup()

wiringpi.wiringPiSetupGpio()

wiringpi.wiringPiSetupPhys()

wiringpi.pinMode(7, 1)

#Getting the I2C address of the LCD

GPIO = PCF.PCF8574(address=0x38)

#Define PCF pins connected to the LCD

lcd_rs		= 4

lcd_en		= 6

d4,d5,d6,d7	= 0,1,2,3

cols,lines	= 16,2

#instantiate LCD display

lcd = Adafruit_CharLCD(lcd_rs, lcd_en, d4, d5, d6, d7,

cols, lines, gpio=GPIO)

lcd.clear()

test = ' '.join(sys.argv[1:])

if len(test)>16:

test = test[0:15] + "\n" + test[15:]

#Display the Contents to the LCD. Turn on Backlight and turn off.

print str(test)

print "LCD ON"

wiringpi.digitalWrite(7,1)

lcd.message(str(test))

sleep(5)

wiringpi.digitalWrite(7,0)

print "LCD OFF"

## 7.2 Application code

Demo

/1 Hardware present?

/1 Memo by student A

/1 Login activity

/1 Data visualization activity

/1 Action control activity

Report

/1 Login activity

/1 Data visualization activity

/1 Action control activity

/1 Modified Code Files in Appendix

/1 Link to Complete Code in Repository